# GETINCELL-LINE® | Double blade louvre – mounted on substructure U 94

Double blade louvre consisting of a substructure [U-shaped carrier] and prefabricated panels.

The specially perforated U-shaped carrier U 94 is suspended rigidly from the mounting level [e.g. bare ceiling] by means of M6 threaded rods using technical approved dowels. The distance between the U-shaped carriers depends on the specified module dimension.

The installation can be done by using the L-shaped primary carrier U 1040 for cross bracing if the suspension height will allow it. The cross bracing is required staring from a suspension height of 500mm. The L-shaped primary carrier is suspended rigidly and secured against loosening from the mounting level [e.g. bare ceiling] by means of M6 threaded rods using technical approved dowels. Afterwards, the U-shaped carrier U 94 is screwed to the L-shaped primary carrier.

The distance between the U-shaped carriers depends on the specified module dimension of the panel element length. The panel elements are mounted at the module spacing and consist of U-shaped sharp-edged blades. The panel element consists of longitudinal bars and crossbars in square cells. The U-shaped, open at the bottom blades are perforated for connecting the panel elements and the U-shaped carriers by means of spring panel hangers [C 94]. The individual panel elements are self-adjusting and can be individually hinged open. A non-interchangeable marking [punched holes] ensures the correct alignment of the panel elements.

Typically, the edge connection is carried out by means of an L-shaped perimeter trim.

All parts are made of pre-coated or anodized aluminium.

| Pos. m² | Panels – GETINCELL-LINE® double blade open cell ceiling Panel size [LxW]:  600x600mm  625x625mm  1200x600mm  1250x625mm Blade width/height:  4/42mm  9/40mm Cell size: ............... mm [66.66-200mm] Perforation:  Microperforation [Ø=1.5mm; diagonal pitch]  Perfo 2.5 [Ø=2.5mm; diagonal pitch] Exterior paint:  White  Black  RAL 9006  RAL 7012 Other colours on request Interior paint:  Green  Blue [on request]  Fuchsia [on request] Other colours on request Material: Aluminium [edges not coated] Unit price: ............… €/m² Total price: ............… € |
| --- | --- |
| Pos. rm | U-profile [U 94] Colour:  White  Black Length: 2400mm Requirement per m²: 0.83rm/m² [for panel 1200x600mm] Unit price: ............… €/rm Total price: ............… € |
| Pos. pcs | U-profile connector [V 94] Colour:  White  Black Requirement: 0.41 pc/m² Unit price: ............… €/pc Total price: ............… € |
| Pos. pcs | Retaining springs [C 94] Requirement: 4 pieces/panel Unit price: …............ €/pc Total price: …............ € |